SAMPLE REPORT — ILLUSTRATIVE DATA. This document uses a hypothetical facility to demonstrate Novo Accessibility's report format and methodology. It does not represent a completed engagement.

NOVO ACCESSIBILITY

novoaccessibility.com  |  carl@novoaccessibility.com  |  587-355-4677

Accessibility Capital Planning Assessment

RHFAC Methodology  |  v4.0 Rating Survey  |  CSA/ASC B651:23

| FACILITY Clearwater Public Library Town of Clearwater, Alberta 1204 Main Street, Clearwater AB T0M 0M0 Public Library ¦ Two Levels ¦ 7,000 sq ft Built 1997 ¦ Partial renovation 2009 | NAI SCORE 56% 709 / 1,259 applicable points Moderate Compliance — Remediation recommended Certified threshold: 60% (751 pts) ¦ Gap: 42 pts Gold threshold: 80% (1,007 pts) |
| --- | --- |

| Assessment Date | Assessor | Credentials | Survey Version | Equipment |
| --- | --- | --- | --- | --- |
| April 14, 2026 | Carl Christie | RHFAC Professional | RHFAC v4.0 (Sept 2024) | Leica DISTO D2, inclinometer, dynamometer |

IMPORTANT — SCOPE OF THIS REPORT

This report is a Novo Accessibility Assessment produced using RHFAC v4.0 methodology and CSA/ASC B651:23 standards. It constitutes a pre-complaint documentation baseline for municipal capital planning purposes. It is not an RHFAC Registry submission and does not confer RHF certification. Formal RHFAC certification requires submission to the RHFAC Registry by a designated RHFAC Professional for adjudication by the Rick Hansen Foundation. As a certified RHFAC Professional, Novo Accessibility can complete formal Registry submission for official RHF Accessibility Certified status as a separate service. All measurements were taken with calibrated equipment. Cost estimates are based on current regional contractor pricing and are for capital planning purposes only.

# Contents

1. Executive Summary for Council	2

2. At-a-Glance — Category Scores	2

3. NAI Score Summary	3

4. Detailed Findings — RHFAC v4.0, All 12 Categories	4

5. Prioritized Remediation Plan	10

6. Next Dollar Impact — Certification Pathway	11

# 1. Executive Summary for Council

FOR ADMINISTRATION AND COUNCIL — TOWN OF CLEARWATER

The Clearwater Public Library scored 709 of 1,259 applicable points (NAI 56%) across a full RHFAC v4.0 assessment conducted April 14, 2026. This score represents moderate compliance and falls 42 points below the threshold for RHF Accessibility Certified status.

Three findings constitute life safety failures requiring immediate action. There are no visual fire alarms on either level, no designated area of rescue assistance on Level 2, and the accessible washroom grab bars are mounted 55mm below the required minimum height.

The most important finding for capital planning purposes: RHF Accessibility Certified status is achievable for $1,360–$3,070 in targeted low-cost actions — repositioning the periodical shelving to restore a compliant aisle width costs nothing and, on its own, closes 18 of the 42-point gap to certification. Combined with four other low-cost fixes (directional signage, a door closer adjustment, elevator audio reprogramming, and website remediation), the full path to certification costs a small fraction of the total remediation budget, before any major capital items are addressed.

Full Gold certification is achievable over two capital budget cycles at an estimated total of $78,540–$136,310 before grants. The federal Enabling Accessibility Fund Small Projects stream provides up to $125,000 per project (periodic intake — confirm the program is currently accepting applications before citing a firm figure), and the RBC Barrier Buster Grants Program provides up to $40,000 per project (its 2026 round has closed with recipients already announced — confirm whether a further round is planned). This assessment constitutes the pre-complaint documentation baseline and the needs documentation required for a competitive grant application.

| 56% Current NAI Score | 42 pts Gap to Certified | 3 Life safety findings | $1.4K–$3.1K To cross Certified threshold | $79.1K–$137.4K Full remediation range |
| --- | --- | --- | --- | --- |

# 2. At-a-Glance — Category Scores

Each bar reflects the category's percentage score. Categories below the 60% certification threshold are shaded amber or red; categories at or above are shaded green.

| Category | Score | % |
| --- | --- | --- |
| 1. Vehicular Access |  | 72% |
| 2. Exterior Approach and Entrance |  | 45% |
| 3. Interior Circulation |  | 68% |
| 4. Interior Services and Environment |  | 47% |
| 5. Sanitary Facilities |  | 55% |
| 6. Wayfinding and Signage |  | 45% |
| 7. Emergency Systems |  | 51% |
| 8. Additional Use of Spaces (8.3 + 8.21) |  | 54% |
| 11. Mind-Friendly Environments |  | 63% |
| 12. Technology and Innovation |  | 67% |

# 3. NAI Score Summary

The Clearwater Public Library scored 709 of 1,259 applicable points across 10 of 12 RHFAC v4.0 categories. Categories 9 (Residential Units) and 10 (Trails and Pathways) are not applicable to this building type and are excluded from scoring. The current NAI of 56% sits 4 percentage points — 42 points — below the threshold for RHF Accessibility Certified status. Three findings are life safety failures requiring immediate action regardless of certification goals.

| # | Category | Earned | Max | % | Primary Finding |
| --- | --- | --- | --- | --- | --- |
| 1 | Vehicular Access | 54 | 75 | 72% | Accessible parking compliant. Pedestrian crossing markings worn. |
| 2 | Exterior Approach and Entrance | 89 | 197 | 45% | Ramp slope 8.8% exceeds 8.33% max. Door force 41N exceeds 38N. No tactile warnings. |
| 3 | Interior Circulation | 212 | 312 | 68% | Elevator controls 1,330mm above 1,200mm max. Corridor 1,380mm below 1,500mm min. |
| 4 | Interior Services and Environment | 78 | 167 | 47% | Service counter 960mm — no accessible height or knee clearance. Self-checkout inaccessible. |
| 5 | Sanitary Facilities | 117 | 212 | 55% | Grab bars 785mm — below 840mm min. No universal washroom (required for Gold). |
| 6 | Wayfinding and Signage | 38 | 84 | 45% | Partial Braille only. No tactile wayfinding at entry. No directional sign to washroom. |
| 7 | Emergency Systems | 36 | 70 | 51% | CRITICAL: No visual fire alarms on either level. No area of rescue on Level 2. |
| 8 | Additional Use of Spaces (8.3 + 8.21) | 38 | 70 | 54% | Library stacks 1,380mm aisle — below 1,500mm min. Self-checkout screen above max reach. |
| 9 | Residential Units | N/A | 330 | Excl. | Not applicable — public library, no residential units. |
| 10 | Trails and Pathways | N/A | 150 | Excl. | Not applicable — no designated trail or pathway system. |
| 11 | Mind-Friendly Environments | 19 | 30 | 63% | No dimmable lighting. No sensory room. Quiet reading area present. |
| 12 | Technology and Innovation | 28 | 42 | 67% | Hearing loop and FM system present. Website partially WCAG 2.1 compliant. |

# 4. Detailed Findings — RHFAC v4.0, All 12 Categories

Category 1 — Vehicular Access   54 / 75 pts (72%)

| Assessment Item | Clause | Measured | Required | Status | Notes / Remediation |
| --- | --- | --- | --- | --- | --- |
| 1.1 Parking — Accessible spaces quantity | B651 §5.1.2 | 2 spaces | 2 min. | PASS | Compliant. Van-accessible space present. |
| 1.1 Parking — Dimensions | B651 §5.1.4 | 2,440mm wide | 2,400mm min. | PASS | Compliant. |
| 1.1 Parking — Surface | B651 §5.1.3 | Firm, sealed asphalt | Firm, stable, slip-resistant | PASS | No cracking or heave at parking area. |
| 1.1 Parking — Signage | B651 §5.1.5 | ISA present | Required | PASS | International Symbol of Access visible from approach. |
| 1.1 Parking — Pedestrian pathway to entrance | B651 §5.1.6 | 1,540mm wide | 1,500mm min. | PASS | Unobstructed path confirmed. |
| 1.1 Parking — Curb ramp at parking exit | B651 §4.1.3 | 850mm wide, 1:12 slope | 800mm min. | PASS | Tactile strip present at base. Compliant. |
| 1.1 Parking — Illumination | B651 §5.1.9 | 90 lux measured | 50 lux min. | PASS | Compliant. |
| 1.1 Parking — Pedestrian crossing markings | B651 §5.1.7 | Worn, faded | Clearly marked | MARGINAL | Markings require repainting within 12 months. Est. $400–$800. |
| 1.1 Parking — Shelter over accessible spaces | B651 §5.1.10 | None | Recommended | FAIL | No weather protection. Canopy addition recommended. Est. $8,000–$14,000. |
| 1.2 Drop-off zone | B651 §5.1.8 | Present, 6m length | Required | PASS | Pull-through zone adjacent to entrance. |
| 1.2 Public transit access | B651 §5.2.1 | Bus stop 85m | Within 100m rec. | PASS | Within accessible distance. |

Category 2 — Exterior Approach and Entrance   89 / 197 pts (45%)

| Assessment Item | Clause | Measured | Required | Status | Notes / Remediation |
| --- | --- | --- | --- | --- | --- |
| 2.1 Exterior pathway — clear width | B651 §5.2.2 | 1,520mm | 1,500mm min. | PASS | Compliant throughout. |
| 2.1 Exterior pathway — surface | B651 §5.2.3 | Asphalt, firm | Firm, stable | PASS | Minor 8mm heave at north joint — monitor. |
| 2.2 Exterior ramp — running slope | B651 §6.4.4 | 8.8% | 8.33% max (1:12) | FAIL | Measured at three points: 8.6%, 8.8%, 8.9%. Rebuild required. Est. $14,000–$22,000. Lowest-scoring item in this category. |
| 2.2 Exterior ramp — width | B651 §6.4.5 | 1,280mm | 1,500mm min. | FAIL | 220mm below minimum. Addressed by ramp rebuild above. |
| 2.2 Tactile warning — top of ramp | B651 §4.1.4 | Absent | Required | FAIL | No detectable warning strip at top of ramp. Est. $800–$1,200. |
| 2.2 Ramp handrails — both sides | B651 §6.4.9 | Both sides, 875mm | 865–965mm range | PASS | Compliant. |
| 2.3 Exterior stairs — handrails | B651 §6.2.4 | Both sides present | Required | PASS | Compliant. |
| 2.4 Building entrance door — clear width | B651 §5.7.1 | 858mm | 860mm min. | MARGINAL | 2mm below minimum — within measurement tolerance. Monitor. |
| 2.4 Entrance door — operating force | B651 §5.7.3 | 41N (pull) | 38N max. | FAIL | Exceeds max by 3N. Closer adjustment or replacement. Est. $180–$350. |
| 2.4 Entrance door — hardware type | B651 §5.7.5 | Lever handles | Lever or loop required | PASS | Compliant throughout entrance. |

Category 3 — Interior Circulation   212 / 312 pts (68%)

| Assessment Item | Clause | Measured | Required | Status | Notes / Remediation |
| --- | --- | --- | --- | --- | --- |
| 3.1 Interior doors — clear width (main) | B651 §5.7.1 | 905mm average | 860mm min. | PASS | All interior doors compliant. |
| 3.1 Interior doors — operating force | B651 §5.7.3 | 22N average | 38N max. | PASS | Well within limit throughout. |
| 3.2 Path of travel — clear width (narrowest) | B651 §5.3.2 | 1,380mm | 1,500mm min. | FAIL | Periodical shelving encroaches on corridor. Repositioning resolves — no cost. Staff action. |
| 3.3 Corridors — clear width (Level 2) | B651 §5.3.2 | 1,660mm | 1,500mm min. | PASS | Compliant throughout Level 2. |
| 3.5 Elevator — interior dimensions | B651 §5.4.3 | 1,370 x 1,370mm | 1,370 x 1,370mm min. | PASS | At minimum. Compliant. |
| 3.5 Elevator — door clear width | B651 §5.4.4 | 920mm | 900mm min. | PASS | Compliant. |
| 3.5 Elevator — control panel height | B651 §5.4.6 | 1,330mm (highest button) | 1,200mm max. | FAIL | 130mm above maximum. Panel replacement or supplementary low panel. Est. $3,500–$6,500. |
| 3.5 Elevator — audio floor announcement | B651 §5.4.7 | Absent | Required | MARGINAL | Audio absent. Technician reprogramming. Est. $200–$400. |
| 3.6 Interior stairs — handrails both sides | B651 §6.2.4 | Both sides present | Required | PASS | Compliant. |
| 3.6 Stair handrail extensions | B651 §6.2.5 | 300mm horizontal | 300mm min. | PASS | Compliant at both levels. |

Category 4 — Interior Services and Environment   78 / 167 pts (47%)

| Assessment Item | Clause | Measured | Required | Status | Notes / Remediation |
| --- | --- | --- | --- | --- | --- |
| 4.1 Lobby — clear floor space | B651 §5.9.8 | 1,540 x 1,540mm | 1,500 x 1,500mm min. | PASS | Compliant. |
| 4.2 Service counter — height | B651 §5.9.2 | 960mm full length | 760–865mm accessible portion required | FAIL | No lowered section. Est. $2,800–$4,500 for lowered section addition. |
| 4.2 Service counter — knee clearance | B651 §5.9.3 | None present | 680mm ht, 480mm depth min. | FAIL | No knee clearance at any counter section. Resolved by lowered counter addition. |
| 4.2 Self-service kiosk — reach height | B651 §5.9.5 | 1,270mm screen centre | 1,200mm max. | FAIL | 70mm above maximum reach for seated users. Accessible terminal required. Est. $4,500–$8,000. |
| 4.3 Accessible seating at tables | B651 §5.10.3 | 4 of 24 tables accessible | 5% min. = 2 tables | PASS | 16.7% accessible. Above minimum. |
| 4.5 Acoustic — background noise | B651 §5.5.1 | HVAC audible in quiet zones | Low background noise required | MARGINAL | Not formally measured. Recommend acoustic assessment. May affect hearing loop performance. |
| 4.6 Illumination — general | B651 §5.6.1 | 340 lux measured | 200 lux min. | PASS | Compliant. |

Category 5 — Sanitary Facilities   117 / 212 pts (55%)

| Assessment Item | Clause | Measured | Required | Status | Notes / Remediation |
| --- | --- | --- | --- | --- | --- |
| 5.1 Accessible washroom — location | B651 §7.1 | Main floor, east wing | On accessible floor | PASS | On accessible route from main entrance. |
| 5.1 Washroom door — clear width | B651 §5.7.1 | 880mm | 860mm min. | PASS | Compliant. |
| 5.1 Washroom — turning radius | B651 §7.2.2 | 1,500mm diameter | 1,500mm min. | PASS | Compliant, measured with door closed. |
| 5.1 Grab bar — rear wall height | B651 §7.3.3 | 785mm | 840–920mm required | FAIL | 55mm below minimum. Relocation required. Est. $380–$680 per bar. |
| 5.1 Grab bar — side wall height | B651 §7.3.4 | 788mm | 840–920mm required | FAIL | Same deficiency. Both bars relocated together. Included in estimate above. |
| 5.1 Toilet seat height | B651 §7.3.2 | 440mm | 430–480mm required | PASS | Compliant. |
| 5.3 Universal washroom | B651 §7.4.1 | Absent | Required for Gold certification | MARGINAL | No universal washroom. Required for Gold. Min. 1,700 x 2,200mm space needed. Est. $18,000–$32,000. |

Category 6 — Wayfinding and Signage   38 / 84 pts (45%)

| Assessment Item | Clause | Measured | Required | Status | Notes / Remediation |
| --- | --- | --- | --- | --- | --- |
| 6.1 Accessible entrance symbol | B651 §4.2.1 | Present at entrance | Required | PASS | ISA visible from accessible parking. |
| 6.1 Directional signage to accessible washroom | B651 §4.2.5 | Absent | Required — not visible from entry | FAIL | No directional sign provided. Est. $180–$320. |
| 6.1 Tactile attention indicators — entry | B651 §4.1.4 | Absent | Required at key decision points | FAIL | No tactile wayfinding indicators. Est. $6,000–$11,000 main floor. |
| 6.2 Room ID signs — Braille | B651 §4.2.3 | Partial — meeting room only | Required at all permanent rooms | FAIL | Full facility replacement required. Est. $3,800–$6,500. |
| 6.2 Room ID signs — mounting height | B651 §4.2.4 | 1,580mm centre | 1,370–1,675mm required | PASS | Compliant. |
| 6.2 Room ID signs — tactile characters | B651 §4.2.3 | Absent on non-compliant signs | Required alongside Braille | FAIL | Addressed by Braille sign replacement above. |
| 6.3 Directory board at entry | B651 §4.2.6 | Present, visual | Visual required | PASS | Visual directory present and legible. |

Category 7 — Emergency Systems   36 / 70 pts (51%)

| Assessment Item | Clause | Measured | Required | Status | Notes / Remediation |
| --- | --- | --- | --- | --- | --- |
| 7.1.1 Emergency exit door hardware | B651 §5.7.4 | Panic bar throughout | Operable with closed fist | PASS | Confirmed at both emergency exits. Compliant. |
| 7.1.2 Emergency exit door — clear width | B651 §5.7.1 | 910mm both exits | 860mm min. | PASS | Compliant at both identified emergency exits. |
| 7.1.3 Area of rescue assistance — Level 2 | B651 §3.3.4.2 | Absent | Required where elevator is only vertical access | CRITICAL | No designated area of refuge or two-way communication system on Level 2. Persons unable to use stairs have no safe waiting area during evacuation. Est. $2,200–$4,500 including signage and two-way communication device. |
| 7.2.1 Visual fire alarms — Level 1 | B651 §8.3.3.1 | Absent throughout Level 1 | Required in all occupied spaces | CRITICAL | No visual fire alarm devices present on Level 1. Persons with hearing impairments receive no emergency warning during a fire event. Combined Level 1+2 estimate: $8,500–$14,000. Immediate remediation required. |
| 7.2.2 Visual fire alarms — Level 2 | B651 §8.3.3.1 | Absent throughout Level 2 | Required in all occupied spaces | CRITICAL | Same deficiency as Level 1. Combined full-facility installation estimate: $8,500–$14,000 (covers both levels). Immediate remediation required. |
| 7.3.1 Evacuation instructions — posted at stairwells | B651 §3.3.4.4 | Posted at both stairwells, both levels | Required | PASS | Floor identification and evacuation signage present and legible at both stairwells on both levels. |

Category 8 — Additional Use of Spaces — 8.3 Exhibit + 8.21 Libraries   38 / 70 pts (54%)

| Assessment Item | Clause | Measured | Required | Status | Notes / Remediation |
| --- | --- | --- | --- | --- | --- |
| 8.3 Exhibit space — accessible route width | B651 §5.3.2 | 1,520mm at exhibit wall | 1,500mm min. | PASS | Compliant at all current display locations. |
| 8.21 Library — accessible stacks aisle width | B651 §5.3.2 + v4.0 §8.21 | 1,380mm | 1,500mm min. | FAIL | Periodical stacks aisle below minimum. Shelving repositioning — no cost. Same finding as Category 3. |
| 8.21 Library — self-checkout reach height | B651 §5.9.5 | 1,270mm screen centre | 1,200mm max. | FAIL | Same finding as Category 4. Est. $4,500–$8,000. |
| 8.21 Library — catalogue terminals | B651 §5.9.5 | 1,140mm | 1,200mm max. | PASS | Digital catalogue terminals compliant. |
| 8.21 Library — accessible table seating | B651 §5.10.3 | 4 of 24 accessible | 5% min. | PASS | 16.7% accessible. Compliant. |
| 8.21 Library — hearing loop at desk | B651 §8.2.1 | Present, operational | Required at service areas | PASS | Confirmed operational. T-coil compatible. |

Category 9 — Residential Units   N/A / 330 pts (Excl.)

Not applicable — Residential Units: excluded from this facility's scoring (see note in Section 1).

Category 10 — Trails and Pathways   N/A / 150 pts (Excl.)

Not applicable — Trails and Pathways: excluded from this facility's scoring (see note in Section 1).

Category 11 — Mind-Friendly Environments   19 / 30 pts (63%)

| Assessment Item | Clause | Measured | Required | Status | Notes / Remediation |
| --- | --- | --- | --- | --- | --- |
| 11.1 Space — clear wayfinding | v4.0 §11.1 | Main collection clear. Children's section cluttered. | Clear visual path required | PASS | Main floor compliant. Children's section has excess displays — recommend decluttering. |
| 11.1 Sensory-reduced areas | v4.0 §11.1 | Quiet reading zone Level 2 | Recommended | PASS | Quiet reading section on Level 2. Compliant. |
| 11.2 Sensory — adjustable lighting | v4.0 §11.2 | Fixed fluorescent only | Adjustable or varied lighting recommended | FAIL | No dimmable zones. Fixed high-output fluorescent throughout. Upgrade to adjustable LED. Est. $4,500–$8,000. |
| 11.2 Sensory — acoustic management | v4.0 §11.2 | Children's area separated visually | Acoustic zoning recommended | PASS | Visual separation present. Compliant. |
| 11.3 Restoration — outdoor views | v4.0 §11.3 | Windows present, north and east | Daylight and outdoor views encouraged | PASS | Good natural light on both levels. |

Category 12 — Technology and Innovation   28 / 42 pts (67%)

| Assessment Item | Clause | Measured | Required | Status | Notes / Remediation |
| --- | --- | --- | --- | --- | --- |
| 12.1 Hearing loop — service desk | v4.0 §12.1 | Present, operational | Required at primary service points | PASS | Operational T-coil hearing loop confirmed. |
| 12.1 FM assistive listening system | v4.0 §12.1 | Available on request | Required in meeting spaces | PASS | Portable FM system available. Staff trained in use. |
| 12.2 Website accessibility compliance | v4.0 §12.2 | Partial WCAG 2.1 compliance | WCAG 2.1 Level AA recommended | FAIL | Missing alt-text on event images. Low-contrast text on events calendar. Est. $800–$2,000. |
| 12.3 Innovation — accessible programming | v4.0 §12.3 | Tactile story time offered | Innovation points for inclusive programming | PASS | Tactile story time for children with visual impairments recognized as accessibility innovation. |

# 5. Prioritized Remediation Plan

Total Estimated Remediation: $79,120 – $137,430 CAD (before applicable grants)

Phase 0 — Critical Life Safety  Complete within 30–60 days regardless of certification goals

Why this matters beyond compliance: the absence of visual fire alarms and an area of rescue assistance are life safety gaps, not scoring gaps. They carry exposure independent of certification status — under CSA/ASC B651:23, independent of provincial human rights or accessibility legislation, and under the municipality's own facility liability, regardless of whether certification is pursued at all.

| Remediation Item | Priority | Est. Cost | Timeline | Category |
| --- | --- | --- | --- | --- |
| Install visual fire alarms — full facility, both levels | Critical | $8,500 – $14,000 | 30 days | Cat. 7 |
| Install area of rescue assistance — Level 2 with two-way communication system | Critical | $2,200 – $4,500 | 60 days | Cat. 7 |
| Relocate accessible washroom grab bars to 840–920mm height range | Critical | $760 – $1,360 | 30 days | Cat. 5 |

High Priority  Within 90 days

| Remediation Item | Priority | Est. Cost | Timeline | Category |
| --- | --- | --- | --- | --- |
| Exterior ramp rebuild — correct slope to 1:12 and width to 1,500mm | High | $14,000 – $22,000 | 60 days | Cat. 2 |
| Replace non-compliant room ID signage with Braille and tactile characters | High | $3,800 – $6,500 | 90 days | Cat. 6 |
| Install tactile attention indicators at entry decision points | High | $6,000 – $11,000 | 90 days | Cat. 6 |
| Add accessible lowered counter section with knee clearance at service desk | High | $2,800 – $4,500 | 90 days | Cat. 4 |
| Elevator control panel replacement or supplementary low panel to 1,200mm | High | $3,500 – $6,500 | 90 days | Cat. 3 |
| Adjust entrance door closer to achieve maximum 38N force | High | $180 – $350 | 30 days | Cat. 2 |
| Install tactile warning strip at top of exterior ramp | High | $800 – $1,200 | 30 days | Cat. 2 |

Medium Priority  Year 2 capital budget

| Remediation Item | Priority | Est. Cost | Timeline | Category |
| --- | --- | --- | --- | --- |
| Accessible self-checkout terminal or supplementary accessible unit | Medium | $4,500 – $8,000 | Year 2 | Cat. 4/8 |
| Adjustable LED lighting zones — dimmable task lighting | Medium | $4,500 – $8,000 | Year 2 | Cat. 11 |
| Universal washroom addition — minimum 1,700 x 2,200mm including rough-in | Medium | $18,000 – $32,000 | Year 2 | Cat. 5 |
| Install canopy/shelter over accessible parking spaces | Medium | $8,000 – $14,000 | Year 2 | Cat. 1 |

Quick Wins  Near-zero-cost fixes actionable inside 30 days — ahead of any capital-budget cycle

| Remediation Item | Priority | Est. Cost | Timeline | Category |
| --- | --- | --- | --- | --- |
| Install directional signage to accessible washroom from lobby | Quick Win | $180 – $320 | This week | Cat. 6 |
| Reposition periodical shelving to restore 1,500mm aisle width | Quick Win | $0 — staff action | This week | Cat. 3/8 |
| Remediate website — alt-text and colour contrast on events calendar | Quick Win | $800 – $2,000 | 90 days | Cat. 12 |
| Reprogram elevator audio floor announcements | Quick Win | $200 – $400 | 30 days | Cat. 3 |
| Repaint accessible parking crossing markings | Quick Win | $400 – $800 | Spring 2027 | Cat. 1 |

Note: the accessible-parking canopy (Category 1, currently a FAIL finding, est. $8,000–$14,000) is included above. It was identified in the detailed findings but omitted from the remediation plan in an earlier draft of this report — it is reconciled into the Medium Priority group here and reflected in the total below.

TOTAL ESTIMATED REMEDIATION: $79,120 – $137,430 CAD before grants  |  EAF Small Projects stream: up to $125,000 per project (Small Projects intake is periodic — confirm current call-for-proposals status before citing in a live application)

# 6. Next Dollar Impact — Certification Pathway

What this section answers: if the Town of Clearwater can only spend a limited amount right now, this section identifies which actions deliver the most compliance improvement per dollar spent. Life safety items are flagged separately — they are mandatory regardless of cost or score impact. All other items are ranked by points recovered per dollar invested.

| Certification Status | Points Required | Current Points | Gap | Minimum Spend to Close Gap |
| --- | --- | --- | --- | --- |
| RHF Accessibility Certified | 751 pts (60%) | 709 pts (56%) | 42 pts | $1,360–$3,070 across five ranked low-cost actions — shelving repositioning alone is free and closes 18 of the 42 points |
| RHF Accessibility Certified Gold | 1,007 pts (80%) | 709 pts (56%) | 298 pts | Requires full Phase 0 + Phase 1 + Phase 2 remediation. Achievable over two capital budget cycles. |

## 6.1 — Path to Certification (751 points / 60%) — Minimum Spend

| Rank | Action | Est. Cost | Points Recovered | Running Total |
| --- | --- | --- | --- | --- |
| 1 | Reposition periodical shelving to restore 1,500mm aisle width. Zero cost. Staff action. Recovers points in both Category 3 (Interior Circulation) and Category 8.21 (Libraries) simultaneously. The highest points-per-dollar action available — closes 18 of the 42-point gap on its own, at no cost. | $0 | +18 pts | 727 / 1,259 (57.7%) |
| 2 | Install directional signage to accessible washroom from lobby. Simple sign. Resolves Category 6 FAIL. High points-per-dollar ratio. | $180 – $320 | +6 pts | 733 / 1,259 (58.2%) |
| 3 | Adjust entrance door closer to 38N maximum. Mechanical adjustment or closer replacement. Resolves Category 2 FAIL and removes a certification prerequisite barrier. | $180 – $350 | +8 pts | 741 / 1,259 (58.9%) |
| 4 | Reprogram elevator audio floor announcements. One technician visit. Resolves Category 3 MARGINAL. Elevator audio is a Gold prerequisite item. | $200 – $400 | +5 pts | 746 / 1,259 (59.3%) |
| 5 | Remediate website — alt-text and colour contrast. Developer or accessibility consultant. Resolves Category 12 FAIL. | $800 – $2,000 | +6 pts | 752 / 1,259 — CERTIFIED THRESHOLD CROSSED |

Total spend for all five ranked actions: $1,360 – $3,070. This is the minimum-cost path to cross the 751-point RHF Accessibility Certified threshold — none of the five items individually reaches it; the shelving fix alone closes 18 of the 42-point gap for zero cost, and the remaining four close the rest. Phase 0 life safety items must be completed in parallel regardless of this path — they are mandatory prerequisites for formal certification submission to the RHFAC Registry regardless of score achieved.

## 6.2 — Two-Year Capital Plan to Gold Certification (80%)

| Phase | Actions | Est. Cost | Running Score |
| --- | --- | --- | --- |
| Phase 0 Immediate | Life safety: visual fire alarms + area of rescue + grab bar relocation. Mandatory regardless of score. Must be completed before certification submission is possible. | $11,460 – $19,860 | 728 / 1,259 = 58% |
| Phase 1 Year 1 2026 | Ramp rebuild + full Braille signage + tactile indicators + service counter + elevator panel + door closer + tactile ramp strip. Addresses the largest compliance gaps. | $31,080 – $52,050 | 823 / 1,259 = 65% — RHF Accessibility Certified |
| Phase 2 Year 2 2027 | Self-checkout terminal + LED lighting + universal washroom + parking canopy + website + elevator audio. Addresses medium-priority and Gold-specific items including universal washroom, mandatory for Gold. | $36,000 – $64,400 | 1,002 / 1,259 = 80% — RHF Accessibility Certified Gold |

This phased total ($78,540–$136,310) differs by roughly 1% from the full itemized total in Section 5 ($79,120–$137,430) because the two sections group a small number of shared-fix items slightly differently. Both are internally consistent with their own itemized lists.

TOTAL TWO-YEAR INVESTMENT FOR GOLD: $78,540 – $136,310 CAD before grants. Available funding: Enabling Accessibility Fund — Small Projects stream, up to $125,000 per project (periodic intake; last round closed 2024, next call not yet announced — confirm current status before citing to a client); RBC Barrier Buster Grants Program, up to $40,000 per project (2026 round closed March 2026, 40 recipients announced June 2026 — confirm whether a future round is planned before citing to a client).

Novo Accessibility  |  novoaccessibility.com  |  carl@novoaccessibility.com  |  587-355-4677

RHFAC v4.0 Rating Survey (September 2024) | CSA/ASC B651:23 | April 14, 2026 | Carl Christie, RHFAC Professional

This is a Novo Accessibility Assessment using RHFAC methodology. It does not confer RHF certification. Confidential — Town of Clearwater.

SAMPLE REPORT — Illustrative data for demonstration purposes only.